# Overview of the CYD ESP32 Module (Cheap Yellow Display)

Back view of the Cheap Yellow Display (CYD) board, showing the ESP32 module, display connector, SD slot, touch controller, and labeled pins.
The ESP32-2432S028R, nicknamed the “Cheap Yellow Display” (CYD), is an ESP32-based development board featuring an integrated 2.8″ 320×240 color TFT touchscreen (ILI9341 driver) with resistive touch panel, a microSD card slot, RGB LED, and other peripherals[1][2]. It contains a dual-core ESP32-WROOM-32 (WiFi/Bluetooth) module (typically 4MB flash, no PSRAM by default) and is powered/programmed via micro-USB[3]. The board is prized for its convenience and low cost (∼$15-$20) since the display and ESP32 come pre-wired on a single PCB[4]. However, using the CYD effectively requires proper setup of libraries and careful handling of the display, touch, and SD interfaces due to some hardware quirks (e.g. SPI bus sharing). Below is a comprehensive guide to methods, configurations, and common issues (screen rotation, sprite rendering, touch calibration, etc.) for developing on the CYD module.

## Setup and Libraries for Display and Touch

Arduino/PlatformIO Setup: The CYD can be programmed with Arduino frameworks (including via PlatformIO) or ESP-IDF. Many projects use Bodmer’s TFT_eSPI library for the TFT and either Paul Stoffregen’s XPT2046_Touchscreen library or a bit-bang driver for the resistive touch. To use TFT_eSPI, you must provide a custom configuration (User_Setup.h) matching the CYD’s pin connections[5][6]. RandomNerdTutorials and the CYD community have published ready-made config files for the CYD to simplify this step[6][7]. Typically, the config will define the TFT’s SPI pins as: MISO=GPIO12, MOSI=GPIO13, SCLK=GPIO14, CS=GPIO15, DC=GPIO2, and TFT Reset = -1 (if the display reset pin is tied or not used)[8][9]. The XPT2046 touch controller is connected to a separate set of SPI pins (by default: T_MOSI=GPIO32, T_MISO=GPIO39, T_SCK=GPIO25, T_CS=GPIO33, T_IRQ=GPIO36)[10][11]. These need to be configured either in code or via library settings. In Arduino, one approach is to instantiate a second SPI bus for touch (as in RandomNerdTutorials’ example) and initialize the XPT2046 with those pins[12][13]. Alternatively – and importantly for using the SD card simultaneously – many developers use the XPT2046_Bitbang library (by @TheNitek) for the touchscreen. This bit-banged driver allows the touch interface to run on arbitrary GPIO pins in software mode, freeing the ESP32’s limited hardware SPI buses for the TFT and SD (more on this under “SPI Bus Sharing”)[14][15].

ESP-IDF or Alternative Libraries: If developing in ESP-IDF, you can either use Arduino components (e.g. run TFT_eSPI in Arduino compatibility mode) or use libraries like LovyanGFX. LovyanGFX natively supports the CYD’s ILI9341 display and XPT2046 touch and is highly optimized. In fact, a community calibration project uses LovyanGFX for the LCD combined with XPT2046_Bitbang for touch (disabling Lovyan’s built-in touch handling) specifically to avoid SPI conflicts[15]. The CYD’s display can also be driven with LVGL (LittlevGL) for sophisticated GUIs – numerous examples exist combining LVGL with TFT_eSPI or LovyanGFX on this board[16][17]. Regardless of library, ensure your software pin assignments match the CYD’s wiring, and remember to configure the backlight pin. On the original CYD, the TFT backlight is controlled by GPIO21 (which doubles as an I2C SDA on the breakout header)[18]. This pin must be set HIGH (output) to turn on the screen. The community notes that if your display stays dark, make sure you are not using GPIO21 for other purposes and initialize it for backlight control[7]. (Some newer board revisions reportedly use GPIO27 for backlight – if you find 21 ineffective, check if 27 responds[19].)

## Screen Rotation and Coordinate Alignment

The ILI9341 display and XPT2046 touch panel each have their own coordinate orientation, so handling screen rotation requires configuring both in sync. The TFT_eSPI library’s tft.setRotation(n) sets the display orientation (n=0..3 for 0°,90°,180°,270°). For example, tft.setRotation(1) sets landscape mode (320×240) on this 240×320 display[20][21]. However, the touch readings will not automatically rotate, so you must adjust them. If using XPT2046_Touchscreen or XPT2046_Bitbang, call touchscreen.setRotation(n) with the same rotation value that you used for the display. In practice, failing to do this leads to X/Y coordinates being swapped or inverted when the screen is rotated[22][23]. For instance, one CYD user discovered that after rotating the TFT to landscape, the touch input was misaligned until they added touchscreen.setRotation(3) alongside tft.setRotation(3) (in their case using rotation=3 for a particular 270° landscape mode)[23]. The rotation index for the touch controller might not always match the display’s exactly (due to the touch panel’s mounting orientation). If touches seem “upside down” or mirrored, try rotation values 0-3 to see which aligns correctly – e.g. CYD guides note some units needed touchscreen.setRotation(3) instead of 1 for proper landscape calibration[24][25].

Even with rotation set, minor axis offsets may exist. You can apply simple calibration by mapping the raw touch coordinates to screen coordinates. For the CYD’s XPT2046, raw ADC values range roughly 0–4095 in each axis, but the meaningful range is smaller (depends on calibration). A basic mapping from raw to pixel can be done via map() once you determine the min/max raw values when touching each extreme of the screen. For example, RandomNerdTutorials found for their unit that X raw ~200 corresponds to left edge and ~3700 to right edge (out of 4095), and Y raw ~240 at top to ~3800 at bottom. They used: x = map(p.x, 200, 3700, 1, 320); y = map(p.y, 240, 3800, 1, 240) to translate raw touch reads to 320×240 screen coordinates[26]. Those numbers can vary by device, so it’s recommended to run a quick test to find your touch’s raw extrema (or use a calibration utility) rather than assuming defaults.

## Precise Touchscreen Calibration

For more precise calibration (critical for accurate small UI elements like keyboard buttons), a linear transformation approach is used to correct both offset and scaling in both axes. The CYD community has developed sketches that have you touch known points (e.g. two or three reference points on the screen) and then compute calibration coefficients. In essence, this yields six coefficients (α_x, β_x, δ_x and α_y, β_y, δ_y) for the linear equations:

X_calibrated = α_y * rawX + β_y * rawY + δ_y  
Y_calibrated = α_x * rawX + β_x * rawY + δ_x

These account for any rotation, axis inversion, and scaling differences. RandomNerdTutorials demonstrates this method, deriving the constants via linear algebra (using a library like BasicLinearAlgebra). They provide an example where the solved coefficients are then applied to every touch reading for corrected output. Using that method, they achieved a calibrated touch where X/Y values were clamped exactly to 0–319 and 0–239 range with proper alignment. The article references a TI application note “Calibration in touch-screen systems” as the basis of the approach[27]. For most use cases, you can start with simpler 2-point calibration (map min/max as described earlier), but for ultimate accuracy the 6-coefficient method can be used. Once you obtain calibration values, you may hardcode them or store them in NVS/Preferences so you don’t need to recalibrate on every reboot[28]. The CYD’s resistive panel has some non-linearity, but calibrating at two well-chosen points can compensate much of it[29].

(Tip: A community calibration tool by Carlo47 uses LovyanGFX + XPT2046_Bitbang to interactively calibrate the CYD touch and save the results in NVS, allowing reuse across projects[15]. This or the RandomNerdTutorials calibration code can serve as “ground truth” references for your own implementation.)

## Sprite Rendering and Performance on ESP32

Sprites (off-screen graphics buffers) in TFT_eSPI are extremely useful for flicker-free animations and game development. You can draw to a TFT_eSprite (which resides in RAM) and then push it to the screen in one call, avoiding the tearing/flicker that comes from drawing many small elements directly to the TFT. The CYD (with an ESP32 WROOM, no PSRAM) can use sprites, but be mindful of memory limits. A full-screen 320×240 sprite at 16-bit color depth requires 3202402 = 153,600 bytes of RAM. In theory the ESP32 has ~520KB of RAM, but in practice contiguous block availability is much smaller (and some memory is reserved), so a 153KB allocation often fails. In fact, Bodmer’s documentation notes that on ESP32 a 16-bit sprite is limited to about 200×200 pixels (~80 KB) reliably[30]. An 8-bit sprite (256-color) uses half the RAM, so you can buffer 320×240 in ~76 KB[31], which just fits in available memory[32]. A 1-bit sprite (monochrome) uses only 9600 bytes for full screen[32] – an option if you only need two colors (e.g. mask or text layer). The key point: Don’t assume you can create a full 16-bit 320×240 sprite on a stock ESP32-WROOM. If you attempt sprite.createSprite(320,240) with default color depth, it may fail or crash due to insufficient contiguous RAM[33][30]. Instead, plan to either use 8-bit depth for large sprites or restrict sprite dimensions and use multiple smaller sprites if needed. For example, one might use a couple of 160×120 or similar regions, or draw portions of the screen in segments. In one Arduino forum discussion, an ESP32 user’s sprite of 320×170 (16-bit) did not work until reducing size and color depth; the advice was to keep total bytes < ~100KB for reliability[34][35]. If you absolutely need full-screen high-color buffering (for complex games), consider using an ESP32 with PSRAM (ESP32 WROVER modules) or even modifying the CYD to add a PSRAM chip (the CYD has an unpopulated footprint intended for PSRAM, although it requires soldering and a minor reroute of an LED pin)[36][37]. Otherwise, optimize by using double buffering only for smaller regions or use the TFT’s fast drawing functions directly with care.

Despite these limits, the ESP32 can still render graphics quite fast over SPI (especially if you use the SPI DMA features in TFT_eSPI). The ILI9341 can often run at 40MHz SPI clock. Just avoid unnecessarily large memory allocations. If flicker is an issue when drawing directly, use sprites for critical UI parts (e.g. a game character or small playfield) rather than the whole screen. Also, note that color depth can be changed: you can create 8-bit sprites by calling createSprite(w, h, 8) and a palette, or even 4-bit/1-bit. This trade-off can make the difference between fitting in RAM or not[30]. In summary, sprite rendering is feasible on the CYD/ESP32 but must respect the memory constraints – a 16-bit sprite over ~80-100KB is likely to exhaust the heap[33]. Use multiple smaller sprites or reduced bit depth to work around this limitation[38].

## Handling Multiple SPI Devices (Display, Touch, SD)

One major “gotcha” with the CYD is that it has three SPI peripherals: the TFT, the touch controller, and the microSD reader. The ESP32, however, has only two usable SPI host controllers (HSPI and VSPI) aside from the one dedicated to flash. In the CYD’s original wiring, each device is connected to separate pins – implying the intent to use three separate SPI buses, which isn’t directly possible in software[39][40]. By default, the display often uses HSPI (e.g. GPIO 12,13,14,15) and the SD card uses VSPI (GPIO 18,19,23,5), leaving no hardware SPI for the touch if it also expects one. This leads to a known issue: “Display, Touch and SD card are not working at the same time.” – as described in the CYD troubleshooting docs[41]. The symptom is that if you try to initialize all three with standard libraries, one of them (usually touch or SD) fails to respond.

Solution 1: Use Software SPI for Touch. The most popular workaround is to run the touchscreen on a bit-banged SPI driver, so it doesn’t consume a hardware SPI channel[14]. The XPT2046_Bitbang library was created for this purpose. By driving the XPT2046 via GPIO toggling in software, it frees up HSPI/VSPI for the TFT and SD. Users report this works well on the CYD – with some code adjustments for the different library’s API – and restores the ability to use the SD card and touch simultaneously[14][42]. In an example shared on Hackaday, a developer ran the display on one SPI bus and SD on the other, and used XPT2046_Bitbang for touch; their project functioned with all three features active after making this change[43]. The CYD community repository explicitly recommends this approach in the troubleshooting guide, noting that the touch library in TFT_eSPI (or using XPT2046 on a second SPI) can conflict, so bit-banging the touch is a reliable fix[44]. If using this library, remember to adjust your pin defines for T_MISO, T_MOSI, T_SCK, T_CS accordingly (as shown in code excerpts above, the CYD’s touch pins are non-standard so you’ll define them manually and call ts.begin() on the bitbang driver instead of touchscreen.begin(SPI) )[45][46]. The XPT2046_Bitbang library also provides its own touch coordinate struct and pressure readings, similar to others, and even has a calibration routine. After switching to it, one user was able to draw UI buttons and read touch and use the SD card without bus conflicts[47][48].

Solution 2: Hardware Mod to Merge Buses. Another solution, if you’re comfortable modding the hardware, is to re-wire the SD and TFT to share the same SPI bus. Mark Stevens published a mod (featured on Hackaday) where he cut and jumpered traces to put the LCD and SD card on the same SPI host (each still with separate CS lines)[49]. This way, you use, say, VSPI for both SD and TFT, and HSPI for touch (or vice versa), staying within the 2-bus limit. It involves cutting a PCB trace and soldering a wire, but nothing too complex[50]. If done correctly, you can then use the standard TFT_eSPI for display and XPT2046_Touchscreen for touch (on the other bus) and initialize both TFT and SD on the shared bus. This approach maintains hardware SPI for all, but note that running the display and SD on one SPI means they share bandwidth (only one can be active at a time). In practice that’s usually fine (you don’t read SD continuously), and SPI devices properly arbitrate via CS lines. The CYD works “out of the box” for many when using bitbang, so that has become the prevalent solution for most software-only fixes[43]. But for a permanent fix without special libraries, the hardware tweak is an option. (Either way, the key is that the ESP32 can only manage two SPI peripherals concurrently; plan your design such that only two hardware SPI devices are active, by either merging lines or offloading one to software.)

Other SPI considerations: The SD card on the CYD is wired for SPI mode (GPIO5 as CS, and sharing MISO/MOSI with either VSPI or HSPI signals depending on revision)[51][18]. This means you’ll use the Arduino SD.h (or SdFat) library, calling SD.begin(SSPin, SPIbus) accordingly. Some users initially forget to initialize the SD with the correct SPI instance or frequency. If you have trouble mounting the SD, double-check you’re using the right CS pin (most found it to be GPIO5, since the board silkscreen labels that) and that no other device is driving the same bus at the moment of init. Also ensure the SD card detect pin (if any) isn’t needed – many boards don’t wire CD, and CYD does not have a card-detect switch to my knowledge.

## Additional Tips, Quirks, and Resources

Backlight Control: As mentioned, GPIO21 controls the LCD backlight transistor on the CYD[18]. The TFT_eSPI library does not automatically manage this pin (since it varies by board), so you may need to do so manually. Typically, set it to OUTPUT and HIGH in setup(). If you accidentally set GPIO21 LOW or use it for I2C without configuring it back to output HIGH, the display will appear off or very dim. This is a common point of confusion – always verify the backlight pin if the screen stays black despite code running[7]. (On some clones where BL is tied to 3.3V through a resistor, this might not apply, but on the official ESP32-2432S028R it is MCU-controlled).

Color Inversion: The ILI9341 display may appear with inverted colors in some orientations or library configs (whites vs blacks swapped). If your display’s colors look wrong (e.g. a white background appears black), you can use tft.invertDisplay(true) (or invertDisplay(1)) to toggle the color inversion[52]. In one example, after setting rotation, a user had to call tft.invertDisplay(1) to get the correct color mode[52]. This might depend on the specific panel variant and orientation, but keep it in mind as a quick fix.

Performance Tweaks: The CYD’s ESP32, running at 240 MHz, can generally drive the ILI9341 at high SPI clock (30–40 MHz). Ensure your TFT_eSPI config sets SPI_FREQUENCY appropriately (e.g. 40 MHz) and SPI_TOUCH_FREQUENCY for touch (typically 2.5 MHz or so for stability). The SD can sometimes run at up to 20 MHz on VSPI. Pushing these frequencies can increase performance (frame rate, etc.), but if you encounter instability (e.g. corrupted SD reads), try lowering the SD or touch SPI speeds slightly. The TFT itself usually handles 40 MHz fine. Also, if using WiFi or other tasks, consider yielding or using vsync strategies if tearing occurs. The LVGL library, if used, has its own buffering and drawing routines optimized for such boards[53][17].

Hardware Limitations: A few hardware caveats were identified by advanced users of the CYD (not all will affect every project): the SD slot is on SPI (not the ESP32’s faster SDMMC 4-bit interface), meaning SD read/write is slower than it could be on a 4-bit bus[40]. The on-board light sensor (LDR) has an incorrect resistor divider in some versions, causing readings to not span the full range – not critical unless you planned to use the LDR. The audio amplifier (if your board has one for a speaker) might clip due to component choices[40]. And as noted, the footprint for a PSRAM chip is present (marked U4 pads on the back) but not populated; adding PSRAM is possible but involves soldering a 8-pin RAM and possibly disconnecting the blue LED on GPIO17 which shares some lines[37][54]. These are advanced modifications – for most developers, the CYD works well enough as is, once the software is configured properly. The board’s popularity means there is a growing community with shared projects.

Resources: Be sure to consult the official community repo “ESP32-Cheap‑Yellow‑Display” by @witnessmenow on GitHub, which hosts examples, pinouts, and a troubleshooting guide[55][56]. Many common questions (blank screen, touch not responding, SD failing) are addressed there. For instance, the guide suggests trying a known-good precompiled project via web-flash to distinguish hardware vs software issues[57]. If that displays correctly, any problem is likely in your code or config (e.g. missing the User_Setup or using the wrong pins)[58]. The RandomNerdTutorials articles on the CYD are also extremely helpful – they cover initial setup, an LVGL GUI example, and even the detailed touchscreen calibration procedure[59][60]. Between those and community forums (Arduino, Reddit, etc.), you can usually find someone who solved a similar issue.

In summary, developing on the CYD module can be very rewarding for IoT interfaces or games once the initial hurdles are overcome. Remember the key points: use the correct pin configuration, sync the touch/display rotations[23], calibrate the touch for accuracy (even a basic map() might suffice)[26], manage your sprite sizes due to RAM limits[61], and handle the SPI bus contention either via bit-bang or hardware tweaks[14]. With these best practices (and the reference sources cited here), you have a “ground truth” to rely on for the CYD’s methods, limitations, and configurations. Happy developing with your Cheap Yellow Display!

Sources: Community and official documentation for the ESP32-2432S028R “Cheap Yellow Display” board – including RandomNerdTutorials setup/calibration guides, the witnessmenow/CYD GitHub documentation, Arduino forum discussions, and Bodmer’s TFT_eSPI notes – were used to compile this comprehensive reference[1][26][43][23][61]. These sources provide further details and example code for the concepts discussed.

[1] [3] [5] [6] [8] [9] [10] [11] [12] [13] [18] [19] [20] [21] [22] [23] [24] [25] [26] [51] [52] ESP32 Cheap Yellow Display Board (ESP32-2432S028R) | Random Nerd Tutorials

https://randomnerdtutorials.com/cheap-yellow-display-esp32-2432s028r/

[2] [4] [55] GitHub - witnessmenow/ESP32-Cheap-Yellow-Display: Building a community around a cheap ESP32 Display with a touch screen

https://github.com/witnessmenow/ESP32-Cheap-Yellow-Display

[7] [16] [17] [42] [45] [46] [47] [48] [53] [56] [57] [58] CYD: SD Issues, probably due to Touch pins? - Programming - Arduino Forum

https://forum.arduino.cc/t/cyd-sd-issues-probably-due-to-touch-pins/1280758

[14] [36] [37] [39] [40] [43] [44] [49] [50] [54] Three SPI Busses Are One Too Many On This Cheap Yellow Display | Hackaday

https://hackaday.com/2025/03/20/three-spi-busses-are-one-too-many-on-this-cheap-yellow-display/

[15] [29] GitHub - Carlo47/CYDtouchCalibration: Calibrating the touchpad of the CYD

https://github.com/Carlo47/CYDtouchCalibration

[27] [28] [59] [60] ESP32 Cheap Yellow Display (CYD) Resistive Touchscreen Calibration | Random Nerd Tutorials

https://randomnerdtutorials.com/esp32-cheap-yellow-display-cyd-resistive-touchscreen-calibration/

[30] [31] [32] [33] [34] [35] [38] [61] Trying to understand Sprites in TFT_eSPI - Displays - Arduino Forum

https://forum.arduino.cc/t/trying-to-understand-sprites-in-tft-espi/1413272

[41] ESP32-Cheap-Yellow-Display/TROUBLESHOOTING.md at main

https://github.com/witnessmenow/ESP32-Cheap-Yellow-Display/blob/main/TROUBLESHOOTING.md